REACH Practical Guide on Exposure Assessment and Communication in the Supply Chains

Examples to Part III: Mixtures under REACH

Case Study 2: Coating

This example illustrates the use of the DPD+ approach to identify lead substances for a substance-rich mixture used as coating.

March 2010

Version: 1

Copyright

Copyright © of Cefic (AISBL) and VCI e.V., reproduction is authorised, except for commercial purposes, provided that the source is mentioned and acknowledged. Cefic and VCI claim no copyright on any official document, such as the REACH guidance documents (whose abstracts may be used in this publication) or information provided by the EU Institutions and ECHA.

Disclaimer

The information given in this REACH Guidance is intended for guidance only and whilst the information is provided in utmost good faith and has been based on the best information currently available, is to be relied upon at the user’s own risk. No representations or warranties are made with regards to its completeness or accuracy and no liability will be accepted for damages of any nature whatsoever resulting from the use of or reliance on the information. This does not apply if damage was caused intentionally or by gross negligence by Cefic or VCI or by parties assisting them.

Important note to the reader:

This case study has been prepared by the author within a VCI working group as part of the joint Cefic/VCI project to develop tools and guidances for industry – in respect of Chemical Safety Assessments, Chemical Safety Reports and Exposure Scenarios.

The REACH Practical Guide on Exposure Assessment and Communication in the Supply Chains developed in this project comprises several parts. An overview is given in the preface of Part I.

The structure and the contents of the REACH Practical Guide are described on the following web sites:

VCI: http://www.vci.de/default~cmd~shd~docnr~125022~lastDokNr~102474.htm

CEFIC: http://cefic.org/templates/shwPublications.asp?HID=750

All related documents can be downloaded from this site. In addition you find there information on related issues and actual developments.

These case studies have been prepared for REACH – related trainings purposes and educational purposes. It is not intended for the fulfilling of legal requirements. Case studies can be used as examples also by third parties if the source is cited.

It is mentioned in the description of the individual example if specific priorities have been set for the preparation of the example.

Author and contact person for this example:

Marco Morabito, Thomas May; Du Pont Performance Coatings

## The background: Preparing safety data sheets and exposure scenarios for a mixture

The following example illustrates steps in the process of assessing a mixture and preparing the related exposure scenario and the extended safety data sheet for the mixture.

The main steps in this process are shown in flowchart 1. It includes the use of existing knowledge, the requirements for the classification and labelling of a mixture and also the new obligations under REACH. Flowchart 1 shows the whole process from the identification of the substance profile of the mixture and its hazard assessment until the preparation of the safety data sheet of the mixture.

A general description of the working steps is given in Part III, chapter 6.1 of the REACH Practical Guide.

Note: It has not been the aim of these case studies to prepare complete chemical safety assessments for the substances in the mixtures. Examples for chemical safety assessments and the related chemical safety reports can be found in Part V of the REACH Practical Guide.

Why we have chosen a mixture used for coating processes as an example?

A coating has been selected as example because

- coatings typically are complex mixtures with 15 to 25 raw materials and 30 to 50 substances. The selected example consists of 18 raw materials and 40 substances.

- hazards of formulations may significantly deviate from hazards of compounds

- substance disclosures in raw material SDS often show gaps (substances below disclosure threshold or not classified, physical and environmental hazards not fully assessed, composition of technical solvent grades not completely known)

- coatings most often are applied different from supply form (mixing of tints, addition of hardener, adjustment of viscosity by dilutants)

- most coatings are sprayed and generate a risk in form of aerosols which cannot be derived from chemical hazards.

Which working steps are covered in the example?

In the example of the coating the working following steps have been performed:

- Identify substances of the mixture and collect existing knowledge

- Document modifications of the determinants of exposure of the mixture

- Carry out classification and labelling (C&L) of the mixture

- Document differences between classification of the mixture and classification of the substances in the mixture.

- Check: Identification of lead substances useful?

- Identify lead substances

- Collect required information for exposure scenario of the mixture / for the safety data sheet of the mixture

Flow chart 1: Assessment of a mixture and generation of an extended safety data sheet

In addition to the task indicated above, the example of the coating illustrates

- the preparation of a costumer information for a ready-to-use mixture (this is not legally required)

- options for advanced evaluation of mixtures.

The working steps and the results

| 1 | Identify substances of the mixture and collect existing knowledge |
| --- | --- |

Table 1 shows the raw materials used for the mixture

Table (1): Raw materials for the mixture

| Raw Material Name | Value |  |
| --- | --- | --- |
| ALUMINUM SILICATE | 22,0000 | % |
| Acrylic resins, curing at ambient temp. | 18,1000 | % |
| BUTYL ACETATE | 12,2812 | % |
| TITANIUM DIOXIDE | 11,0000 | % |
| Anticorrosive pigment | 9,8000 | % |
| TALC, MICRONIZED | 7,7000 | % |
| NITRO CELLULOSE, moistened | 7,0000 | % |
| XYLENE | 3,1700 | % |
| ETHOXYPROPYL ACETATE | 2,9000 | % |
| ZINC OXIDE | 2,9000 | % |
| BLACK IRON OXIDE | 1,1000 | % |
| MAGNESIUM MONTMORILLONITE | 0,7000 | % |
| CARBOXYLIC ACID SALT | 0,7000 | % |
| AROMATIC HYDROCARBON | 0,4900 | % |
| IRON OXIDE YELLOW | 0,1000 | % |
| POLYETHER MODIFIED DIMETHYLPOLYSILOXANE CO | 0,0300 | % |
| CROSSLINKER | 0,0188 | % |
| DISPERSING ADDITIVE | 0,0100 | % |
|  | 100,0000 | % |

Table 2 shows the substances in the mixture

Table (2): Substances in the mixture

| Substance Name | Value |  | CAS Number |
| --- | --- | --- | --- |
| KAOLIN | 21,782 | % | 1332-58-7 |
| BUTYL ACETATE | 16,809 | % | 123-86-4 |
| Acrylic resins, curing at ambient temp. | 13,032 | % | 83770-98-3 |
| TITANIUM DIOXIDE | 10,373 | % | 13463-67-7 |
| ZINC PHOSPHATE | 9,800 | % | 7779-90-0 |
| HYDROUS MAGNESIUM SILICATE | 4,766 | % | 14807-96-6 |
| XYLENE | 4,190 | % | 1330-20-7 |
| MAGNESITE | 2,926 | % | 546-93-0 |
| ZINC OXIDE | 2,898 | % | 1314-13-2 |
| ETHOXYPROPYL ACETATE | 2,693 | % | 98516-30-4 |
| NITROCELLULOSE | 1,610 | % | 9004-70-0 |
| AROMATIC HYDROCARBON | 1,484 | % | 64742-95-6 |
| BLACK IRON OXIDE | 1,073 | % | 1317-61-9 |
| ETHYLBENZENE | 1,042 | % | 100-41-4 |
| METHOXY BUTYL ACETATE | 1,014 | % | 4435-53-4 |
| 1,2,4-TRIMETHYL BENZENE | 0,950 | % | 95-63-6 |
| N-BUTYL ALCOHOL | 0,845 | % | 71-36-3 |
| HECTORITE CLAY | 0,700 | % | 71011-27-3 |
| ALUMINUM HYDROXIDE | 0,473 | % | 21645-51-2 |
| WATER | 0,224 | % | 7732-18-5 |
| 1,3,5-TRIMETHYL BENZENE | 0,214 | % | 108-67-8 |
| SALT OF AMINE CARBOXYLIC ACIDS | 0,210 | % | - |
| PROPANOL, 1(OR 2)-ETHOXY-, ACETATE | 0,203 | % | 98516-30-4 |
| AMORPHOUS SILICA | 0,182 | % | 7631-86-9 |
| SALT OF AMINE CARBOXYLIC ACIDS | 0,140 | % | - |
| BENZENE, PROPYL- | 0,114 | % | 103-65-1 |
| IRON HYDROXIDE | 0,100 | % | 20344-49-4 |
| CUMENE | 0,050 | % | 98-82-8 |
| TOLUENE | 0,031 | % | 108-88-3 |
| DIBUTYL TIN DILAURATE | 0,019 | % | 77-58-7 |
| POLYSILOXANE | 0,010 | % | - |
| GLYCOLS, POLYETHYLENE POLYPROPYLENE, MONOBUTYL ETHER | 0,009 | % | 9038-95-3 |
| ISOBUTYL ACETATE | 0,008 | % | 110-19-0 |
| QUARTZ-CRYSTALLINE SILICA | 0,008 | % | 14808-60-7 |
| NZTR_8056 | 0,007 | % | - |
| ISOBUTYL ALCOHOL | 0,004 | % | 78-83-1 |
| PROPYLENE GLYCOL MONOMETHYL ETHER ACETATE | 0,003 | % | 108-65-6 |
| ISOAMYL ACETATE | 0,002 | % | 123-92-2 |
| LEAD OXIDE SULFATE | 0,002 | % | 12202-17-4 |
| POLYMER | 0,001 | % |  |
|  | 100,000 | % |  |

Some of the substances in the mixture might be not relevant for the assessment because they are (1) below concentration limits of DPD / REACH (0.1%, 1% or substance specific limit values, see REACH Art. 14.2) or they are (2) non dangerous / non PBT / non vPvB. Application of these criteria can reduce the number of substances which is further considered.

In the example of the coating, only 9 substances are relevant for the assessment. These substances are shown in table 3.

Table (3): Substances relevant for the assessment of the mixture

| Substance Name | Value |  | CAS Number |  |
| --- | --- | --- | --- | --- |
| BUTYL ACETATE | 16,809 | % | 123-86-4 |  |
| ZINC PHOSPHATE | 9,800 | % | 7779-90-0 |  |
| XYLENE | 4,190 | % | 1330-20-7 |  |
| ZINC OXIDE | 2,898 | % | 1314-13-2 |  |
| ETHOXYPROPYL ACETATE | 2,693 | % | 98516-30-4 |  |
| AROMATIC HYDROCARBON | 1,484 | % | 64742-95-6 |  |
| ETHYLBENZENE | 1,042 | % | 100-41-4 |  |
| 1,3,5-TRIMETHYL BENZENE | 0,214 | % | 108-67-8 |  |
| BENZENE, PROPYL- | 0,114 | % | 103-65-1 |  |

Some of the raw materials contain technical solvent blends (solvent naphtha or similar) which are composed of a subset of substances needing disclosure. However, some of the raw material suppliers did not supply appropriate component breakdown information. This is a very common behaviour of some distributors.

Nitrocellulose typically entails a hazard of ignition at relatively low temperatures. This hazard is expected to be described in information about this specific raw material. As in the finished mixture, nitrocellulose content is below 5 %, this hazard descriptor is not applicable finally.

In addition, it is desirable to share information about known impurities which may have an impact for the aquatic environment, e.g. release of lead or AOX generators.

| 2 | Document modifications of the determinants of exposure of the mixture |
| --- | --- |

For many mixtures, formulators make an expert judgement on characteristics of the mixture (Physicochemical properties, human health properties, environmental properties). It is based on their knowledge of the mixture.

The assessment of the coating is based on

- testing of the mixture itself and

- properties of the substances

For the C & L of the coating, the following tests have been performed with the mixture:

Physico-chemical data as shown in chapter 9 of SDS

| 3 | Carry out classification and labelling (C&L) of the mixture |
| --- | --- |

The result of C&L is documented in the following excel file, sheet 1, column C.

| 4 | Document differences between classification of the mixture and classification of the substances in the mixture. |
| --- | --- |

Such differences indicate that information from exposure scenarios of raw materials might need modification before they can be used for the exposure scenario of the mixture.

The classification of the mixtures differs from the classification of the raw materials. This is documented in the  excel, file included above, sheet 1, column C.

All pigments, extenders and solid additives are bound in a polymer matrix. This influences the risk assessment of the mixture, e.g. the hazard associated with crystalline silica disappears.

| 5 | Check: Identification of lead substances useful? |
| --- | --- |

The lead substance approach is useful for substance-rich mixtures if the hazard assessment of the mixtures is based on data of the substances. If the hazard assessment is testing of the mixture or on expert knowledge, identification of lead substances is not required.

Identification of a lead substances has been useful for inhalative, dermal and environmental exposure.

| 6 | Identify lead substances and priority substances |
| --- | --- |

A description of the methodology to identify lead substances is given in Part III, chapter 7.2 of the REACH Practical Guide.

Lead substances were identified using DPD+ as described in the REACH Practical Guide.

The input information and the results are shown in the following document.

Lead compounds of  the coating identified by DPD+ were:

Inhalative exposure: Butylacetate

Dermal exposure: Xylene

Eye exposure: n-Butyl alcohol

Ingestion: Aromatic hydrocarbon

Aquatic Environment: Zinc phosphate

Advanced evaluation: Does your mixture contain priority substances which are not covered by the DPD+ method (e.g. CMR 1 + 2 substances, PBT/vPvB substances) in relevant concentrations which require an advanced evaluation of the mixture?

Isocyanate prepolymers are used in the hardener component of the coating. This compound is classified as a skin sensitizer (R 43). According to recommendation of European isocyanate manufacturers, R 42 (inhalative sensitizer) is added for mixtures which are designed for spray application. This represents a condition for advanced evaluation.

Specific operational conditions: Do you have to consider specific operational conditions which require an adjustment of the risk management measures?

Irrespective of the isocyanate issue, spraying is always considered as a specific operational condition which may require respiratory protection equipment even for inert compounds.

| 7 | Collect required information for exposure scenario of the mixture / for the safety data sheet of the mixture |
| --- | --- |

Information on risk management measures and operational conditions for mixtures can be taken from several sources. If no exposure scenarios are available for the lead substances of the mixture, starting point could be safety data sheets of the substances. In addition, sector-specific information on the use of the substances and mixtures could used. RMMS and OCs can be determined by using exposure assessment tools as COSSH essential / EMKG, see Part III, chapter 6.2  of the REACH Practical Guide.

The following information and/or tools have been used to identify adequate risk management measures (RMMs) and operational conditions (OCs):

- Safety data sheets of raw materials

- Sector-specific information on RMMs and OCs

- Own experience in exposure assessment

| 8 | Generate exposure scenario of mixture or integrate information into chapter 1 – 16 of the SDS |
| --- | --- |

8.1/8.2 	Compile and consolidate risk management measures and operational conditions

It might be that information from exposure scenarios of substances has to be adapted to the specific conditions of use in the mixture. See  Part III, chapter 6.3 of the Practical Guide for details.

In the coating example, ES and extended safety data sheets have not been prepared.

In the case of the coating example, we have tried to ensure that all expected uses are covered with reference to CEPE generic ES information. Expectedly, the use of all substances in the mixture will be covered by the exposure scenarios of the substances received from the supplier.

Other examples show that there may be cases where use of some substances in the mixture will not been covered by the exposure scenarios of the substances received from the supplier, e.g. use of crystalline silica in coatings or use of phenolic resins with residual phenol content for spray applications. In these cases a report to ECHA may become necessary.

According to REACH Article 38 the downstream user has to report to ECHA information on uses which are not covered by the exposure scenario of his suppliers (for details see chapter 4.5 of the REACH Practical Guide). In the case studies the focus has been on two items:

- the brief general description of the use(s), as specified in section 3.5 of Annex VI, and

- the conditions of use.

In addition, it has been assessed in the case studies whether it would be required to report a proposal for additional testing to ECHA.

8.3 Check whether identified risk management measures for lead substances cover other substances in the mixture

Priority substances require most stringent risk management measures. However, if these measures are substance-specific (e.g. a precipitation step), it is possible that they do not control adequately further other substances of the mixture with different physicochemical properties. Therefore it is recommended to apply control steps to ensure that the recommended risk management measures are really adequate for the mixture. These steps are shown in Flow Chart 3 in chapter 7.9.4 of the REACH Practical Guide.

Use of sensitizing compounds in the mixture requires additional assessment and results in request for appropriate personal protection equipment (gloves, goggles).

For the coating example, the following aspects require an adjustment of the risk management measures:

- As nitrocellulose is one of the compounds (lightly inflammable solid), risk management measures for manufacturing have to be adjusted. As the nitrocellulose content of the finished product is far below 5 %, this aspect can be neglected for the mixture.

8.4	Decide on scaling guidance and – if yes – develop this guidance

Scaling can broaden the range of applications which are covered by an exposure scenario. Scaling means the use of simple equations in the exposure scenario by which the downstream user can demonstrate that he operates within the conditions of the exposure scenario provided by the registrant. It is described in chapter 8.4 of the REACH Practical Guide.

Guidance on scaling has not been developed in this example.

8.5	Decide in which form the information should be included in the safety data sheet     of the mixture

Downstream users have to include in their own safety data sheets information which they have received from their suppliers (REACH Art. 31.7, see chapter 7.2). They can do this in several ways. They can forward exposure scenarios, they can consolidate information (into an exposure scenario for the mixture) or they can integrate information into chapter 1 – 16  of the safety data sheet (see chapter 7.6 of the REACH Practical Guide).

In the case of the coating, inclusion of information has been done in the following way:

- Consolidation of information into an exposure scenario for the mixture which is annexed to the extended safety data sheet.

- Further interesting aspects of this case study

In addition to the extended safety data sheet for the coating as supplied, an information for the ready for use (RFU) mixture is generated. This is not legally required. Differences between the mixture as supplied and the mixture as applied (RFU) are shown in the following table.

| Raw Material |  | Formulation as supplied | Formulation as applied (RFU) |
| --- | --- | --- | --- |
| XYLENE | 1330-20-7 | 4,2 | 6,2 |
| ETHYLBENZENE | 100-41-4 | 1,0 | 1,5 |
| AROMATIC HYDROCARBON | 64742-95-6 | 1,5 | 2,7 |
| METHOXY BUTYL ACETATE | 4435-53-4 | 1,0 | 0,8 |
| 1,2,4-TRIMETHYL BENZENE | 95-63-6 | 1,0 | 1,8 |
| 1,3,5-TRIMETHYL BENZENE | 108-67-8 | 0,2 | 0,4 |
| BENZENE, PROPYL- | 103-65-1 | 0,1 | 0,2 |
| BUTYL ACETATE | 123-86-4 | 16,8 | 22,0 |
| SALT OF AMINE CARBOXYLIC ACIDS |  | 0,2 | 0,2 |
| SALT OF AMINE CARBOXYLIC ACIDS |  | 0,1 | 0,1 |
| ETHOXYPROPYL ACETATE | 98516-30-4 | 2,9 | 2,2 |
| PROPYLENE GLYCOL MONOMETHYL ETHER ACETATE | 108-65-6 |  | 0,4 |
| ZINC OXIDE | 1314-13-2 | 2,9 | 2,2 |
| ZINC PHOSPHATE | 7779-90-0 | 9,8 | 7,5 |
| NITROCELLULOSE | 9004-70-0 | 1,6 | 1,2 |
| N-BUTYL ALCOHOL | 71-36-3 | 0,9 | 0,7 |
| HDI-Präpolymer | 28182-81-2 | 35,0 | 7,7 |
| HDI-Monomer | 212-485-8 | 0,1 | 0,02 |

The following file documents a safety data sheet for a ready-for-use mixture.