REACH Practical Guide on Exposure Assessment and Communication in the Supply Chains

Examples to Part III: Mixtures under REACH

Case Study 5: Pipe Cleaner

This example illustrates the use of existing knowledge about the properties of a product for the assessment of a mixture and for the building of the exposure scenario.

March 2010

Version: 1

Copyright

Copyright © of Cefic (AISBL) and VCI e.V., reproduction is authorised, except for commercial purposes, provided that the source is mentioned and acknowledged. Cefic and VCI claim no copyright on any official document, such as the REACH guidance documents (whose abstracts may be used in this publication) or information provided by the EU Institutions and ECHA.

Disclaimer

The information given in this REACH Guidance is intended for guidance only and whilst the information is provided in utmost good faith and has been based on the best information currently available, is to be relied upon at the user’s own risk. No representations or warranties are made with regards to its completeness or accuracy and no liability will be accepted for damages of any nature whatsoever resulting from the use of or reliance on the information. This does not apply if damage was caused intentionally or by gross negligence by Cefic or VCI or by parties assisting them.

Important note to the reader:

This case study has been prepared by the author within a VCI working group as part of the joint Cefic/VCI project to develop tools and guidances for industry – in respect of Chemical Safety Assessments, Chemical Safety Reports and Exposure Scenarios.

The REACH Practical Guide on Exposure Assessment and Communication in the Supply Chains developed in this project comprises several parts. An overview is given in the preface of Part I.

The structure and the contents of the REACH Practical Guide are described on the following web sites:

VCI: http://www.vci.de/default~cmd~shd~docnr~125022~lastDokNr~102474.htm

CEFIC: http://cefic.org/templates/shwPublications.asp?HID=750

All related documents can be downloaded from this site. In addition you find there information on related issues and actual developments.

These case studies have been prepared for REACH – related trainings purposes and educational purposes. It is not intended for the fulfilling of legal requirements. Case studies can be used as examples also by third parties if the source is cited.

It is mentioned in the description of the individual example if specific priorities have been set for the preparation of the example.

Author and contact person for this example:

Dr. Dieter Fink

Case Study:  Pipe Cleaner

This example illustrates the use of existing knowledge about the properties of a product for the assessment of a mixture and for the building of the exposure scenario.

## The background: Preparing safety data sheets and exposure scenarios for a mixture

The following example illustrates steps in the process of assessing a mixture and preparing the related exposure scenario and the extended safety data sheet for the mixture.

The main steps in this process are shown in flowchart 1. It includes the use of existing knowledge, the requirements for the classification and labelling of a mixture and also the new obligations under REACH. Flowchart 1 shows the whole process from the identification of the substance profile of the mixture and its hazard assessment until the preparation of the safety data sheet of the mixture.

A general description of the working steps is given in chapter 7.3 of the REACH Practical Guide on Exposure Assessment and Communication in the Supply Chains.

Note: It has not been the aim of these case studies to prepare complete chemical safety assessments for the substances in the mixtures. Examples for chemical safety assessments and the related chemical safety reports can be found in Part V of the REACH Practical Guide.

Why we have chosen a pipe cleaner as an example for a mixture?

The pipe cleaner was selected as a practical example to demonstrate how to elaborate an exposure scenario and the risk assessment / risk management for a mixture on the basis of the existing knowledge of a product. The example includes the assessments for the safety of workers and the safety of the environment for the uses of the formulator as well of the worker safety and environmental safety in the end use. The end use is carried out by private consumers and by professional cleaners.

A further reason has been that a chemical safety report and an extended safety data sheet for sodium hydroxide have been prepared in the first phase of the REACH Practical Guide project.

Which working steps are covered in the example?

In the example of the pipe cleaner the following working steps have been performed:

- Identify substances of the mixture and collect existing knowledge

- Document modifications of the determinants of exposure of the mixture

- Carry out classification and labelling (C&L) of the mixture

- Document differences between classification of the mixture and classification of the substances in the mixture.

- Check, whether DPD+ Methodology can be applied

- Collect required information for exposure scenario of the mixture / for the safety data sheet of the mixture

- Generate exposure scenario for mixture

Flow chart 1: Assessment of a mixture and generation of an extended safety data sheet

The working steps and the results

| 1 | Identify substances of the mixture and collect existing knowledge |
| --- | --- |

Table (1): Raw materials used for the mixture

| 1 | Sodium hydroxide (NaOH) solid |
| --- | --- |
| 2 | Aluminium (Al) powder / swarf |
| 3 | Sodium nitrate (NaNO3) |
| 4 | Sodium carbonate (Na2CO3) |

Table (2): Substances in the mixture (substance, content, R phrases)

| 1 | Sodium hydroxide (NaOH) solid | 60% | R.34 |
| --- | --- | --- | --- |
| 2 | Aluminium (Al) powder / swarf | 5% | R.10; R.15 |
| 3 | Sodium nitrate (NaNO3) | 5% | R.8; R.22; R.36 |
| 4 | Sodium carbonate (Na2CO3) | 30% | R.36 |

All components of the mixture are above the concentration limits.

Hazard assessments of the components:

NaOH solid: 	„corrosive“ (C), R: 34; DNEL inhal.: 2 mg/m3; PNEC: pH = 8 – 9 or 10-5OH- ions

NaNO3: „oxidizing“ (O), R: 8; „harmful“ (Xn), R: 22; „irritant“ (Xi), R: 36;

DNEL: 3 mg/m3; PNEC: -

Al powder: „flammable“, R: 10, 15; DNEL inhal.: 3 mg/m3; PNEC: -

Na2CO3: 	„irritant“, R: 36; DNEL inhal.: 2 mg/m3; PNEC: pH = 8 – 9 or 10-5 OH- ions

This detailed information of the components are provided in the safety data sheets of the substances (raw materials) supplied. The information on the classification is in chapter 16 of the SDS, the DNELs and PNECs are be stated in chapter 8 of the SDS.

If for all the components DNELs and PNECs are available the critical component method may be used as well.

| 2 | Document modifications of the determinants of exposure of the mixture |
| --- | --- |

For many mixtures, formulators make an expert judgement on characteristics of the mixture (Physicochemical properties, human health properties, environmental properties). It is based on their knowledge of the mixture.

The assessment of the pipe cleaner is based on

- testing of the mixture itself and on the

- properties of the substances

The pipe cleaner is formulated as a dust free solid granulate.

Its hazardous properties and other properties had been assessed by experts, by calculation according the Dangerous Preparations Directive DPD and on the basis of testing. The pipe cleaner is classified corrosive.

For the C & L of the pipe cleaner, the following tests have been performed with the mixture:

The flammability of the pipe cleaner was tested (obligation of the DPD). The oxidizing properties were assessed on expert knowledge. The acute toxicity was calculated on the basis of the DPD.

Interactions between the substances which are important for the assessment / additivity of substances for specific endpoints / the use of this knowledge for the assessment of the mixture?

The hazardous properties of the following components

- Aluminium powder

- Sodium nitrate

are not existing in the mixture anymore. The irritant properties of sodium carbonate are negligible compared to the corrosivity of sodium hydroxide. Due to the granulation the hazards / properties in regard of dusts and inhalation are as well not relevant for the pipe cleaner. No exposure by inhalation is to be expected.

| 3 | Carry out classification and labelling (C&L) of the mixture |
| --- | --- |

C + L of mixture

The pipe cleaner has the following classification and labeling:

Corrosive R: 35 „cause severe burns”

| 4 | Document differences between classification of the mixture and classification of the substances in the mixture. |
| --- | --- |

Such differences indicate that information from exposure scenarios of raw materials might need modification before they can be used for the exposure scenario of the mixture.

Result of the modifications:

No flammable properties (Al), no oxidizing properties (NaNO3), no oral toxicity (NaNO3) and no irritant properties (Na2CO3). Only the hazardous properties of sodium hydroxide are prevailing.

| 5 | Check: Identification of lead substances useful? |
| --- | --- |

The lead substance approach is useful for substance-rich mixtures if the hazard assessment of the mixtures is based on data of the substances. If the hazard assessment is testing of the mixture or on expert knowledge, identification of lead substances is not required.

Based on experts knowledge NaOH solid may be regarded as the lead substance for the risk assessment of the pipe cleaner. The mixture itself has in regard of safety the same properties as NaOH solid.

The identification of a lead substance by using the DPD+ method is not appropriate due to the fact that the changes of the physical chemical properties could not be assessed by this method, especially the flammability of aluminium and the oxidizing properties of sodium nitrate. The DPD+ method would identify NaOH solid as lead substance for the dermal exposure route. The DPD+ method would additionally identify sodium nitrate as lead substance for the oral exposure route – however this is not relevant because the mixture is not classified in a class of acute toxicity.

| 6 | Identify lead substances and priority substances |
| --- | --- |

A description of the methodology to identify lead substances is given in Part III, chapter 7.2 of the REACH Practical Guide.

No lead substances and no priority substances were identified.

If DPD+ would be applied following lead substances would be identified. NaOH would be the lead substance for dermal exposure and NaNO3 for oral exposure. The identification of NaNO3 is misleading due to the fact that the pipe-cleaner poses no oral toxicity hazard. For the physical chemical properties “oxidizing” and “flammable” the DPD+ method is not applicable.

Experiences with DPD+:

DPD+ is not appropriate. See above.

Advanced evaluation:

No advanced evaluation needed. The mixture was assessed on the basis of the existing information and expert knowledge without using DPD+. DPD+ would have been only an additional unnecessary step with misleading information.

Specific operational conditions:

The RMM and OC of NaOH solid cover all risks. But for the pipe cleaner only the consumer use and the use of professional cleaners – which may be regarded similar to the consumer use – had to be addressed and not any use which was in the scope of the original NaOH exposure scenario communicated with the SDS of the raw material. For the end use the information is to be reduced to the specific customers.

Changes in exposure:

There are no changes of exposure of the mixture in comparison to the NaOH solid. The RMM and OC had to prevent any skin contact, any relevant emissions into the water and any dust / aerosols.

| 7 | Collect required information for exposure scenario of the mixture / for the safety data sheet of the mixture |
| --- | --- |

Information on risk management measures and operational conditions for mixtures can be taken from several sources. If no exposure scenarios are available for the lead substances of the mixture, starting point could be safety data sheets of the substances. In addition, sector-specific information on the use of the substances and mixtures could used. RMMS and OCs can be determined by using exposure assessment tools as COSSH essential / EMKG, see Part III, chapter 6.2  of the REACH Practical Guide.

The following information and/or tools have been used to identify adequate risk management measures (RMMs) and operational conditions (OCs):

- The CSR and the ES of NaOH solid (this information is already available in the annexes of the REACH Practical Guidance Phase 1).

- The exposure scenario for sodium hydroxide (solid) could be used directly. Only the language of the exposure scenario might be changed in order to achieve a better understanding by the private customers and the professional cleaners.

| 8 | Generate exposure scenario of mixture or integrate information into chapter 1 – 16 of the SDS |
| --- | --- |

8.1/8.2 	Compile and consolidate risk management measures and operational conditions

In order to generate the exposure scenario for the mixture, the relevant pieces of  information from the exposure scenarios of the identified lead substances have been taken and combined to the exposure scenario of the mixture.

The exposure scenario of NaOH solid covers all risk management measures and conditions of use which are relevant for the pipe cleaner. But the OC and RMM which relate to caustic soda (NaOH + H2O) are not relevant (e.g. viscosity). The information on the amounts used and the frequency / duration of the use of the pipe cleaner has been specified.

According to REACH Article 38 the downstream user has to report to ECHA information on uses which are not covered by the exposure scenario of his suppliers (for details see chapter 4.5 of the REACH Practical Guide). In the case studies the focus has been on two items:

- the brief general description of the use(s), as specified in section 3.5 of Annex VI, and

- the conditions of use.

In addition, it has been assessed in the case studies whether it would be required to report a proposal for additional testing to ECHA.

Information on use and conditions of use: (information which might be notified to ECHA)

Industrial-, professional- and consumer use of pipe cleaner granulate. Descriptors: SU 10, SU 21, SU22, PC 35, PROC 5- (but without significant exposure) – not obligatory.

- industrial filling into packages for the end-use,

- storage of the packages

- end-use by professional cleaners and private consumers

- pouring of pipe cleaner into drain pipes / sinks

Proposal on animal testing:

No proposal on animal testing. The information available on the mixture is sufficient for safe use.

8.2 Preparation of DU CSA / Notification to ECHA:

The downstream user would have to document that he deviates from the exposure scenarios he received for aluminium and sodium nitrate. In regard of the exposure scenario of sodium carbonate he implements / recommends a more stringent exposure scenario. He has to document this as well. He is obliged to make a notice to ECHA in regard of the ES of aluminium and sodium nitrate.

He does not need to elaborate a new hazard assessment because he could use the hazard assessment of sodium hydroxide, which did not include the inhalation exposure (if the sodium hydroxide ES would already include conditions of use for the inhalation exposure he nevertheless would have to report the deviation of the conditions of use as well to ECHA).

8.3 Compilation of the RMM for the pipe cleaner

3. Control of workers / consumer exposure:

a) Instructions:

- No skin / eye contact – touching forbidden (for consumers and workers),

- use protective gloves and goggles

- avoid any dust / no use without LEV if dust arises (for workers in the formulation area)

- keep out of reach of children

b) Product related measurements:

- formulate/ produce only dust free granulates,

- Warning: water may cause dangerous reactions in pipe cleaner packages

- packaging with child resistant closures only

c) Organisation measures: e.g.

- use only according to instructions

- regular control of compliance with instructions (workers)

- check of efficiency of the RMM (workers)

d) Technical measures:

e.g. transport in closed containers

e) Personal measures:

protective clothes, goggles, gloves

4. Control of environmental exposure:

a) Instructions:

- No emissions in water without neutralizing (only for industrial use)

- residuals in containers: clean / dilute with much water

c) Organisational measures:

- regular control of pH value: 8 – 9 (industrial use – formulating)

5. Waste: no special measures.

No adjustment of the RMM is necessary in comparison to NaOH solid.

8.4	Decide on scaling guidance and – if yes – develop this guidance

No scaling necessary.

8.5	Decide in which form the information should be included in the safety data sheet     of the mixture

For the pipe cleaner the integration of information into chapter 1 – 16 of the extended safety data sheet was decided to be the most appropriate. There is no need to amend the existing safety data sheets extensively. Only minor changes are necessary for the better understanding. The customers (professional cleaners) would not be able to understand a structured exposure scenario based on the templates of ECHA. They have no knowledge how to cope with the descriptors neither they are able to understand the assessed levels of exposure based on certain assessment tools like ECETOC. The specific RMMs and OCs are the only relevant information for them.

  - Document the result: exposure scenario and / or extended safety data sheet of the mixture

The formalistic exposure scenario which covers all formal aspects for the pipe cleaner even if not relevant, which would be appropriate for the registration is the following:

1. Short title:

Industrial, professional and consumer use of pipe cleaner granulate. Descriptors: SU 10, SU 21, SU 22, PC 35, PROC 5 - but without significant exposure (not obligatory).

- industrial filling into packages for the end-use,

- storage of the packages

- end-use by professional cleaners and private consumers

- pouring of pipe cleaner into drain pipes / sinks

2. Operational conditions and risk management measures:

- Product characteristics:

Solid granulates; no / minimized dustiness

- Amounts used / frequency, duration:

Pipe cleaner is occasionally used at 50-60 g per application (< 1 min)

3. Control of workers / consumer exposure:

a) Instructions:

- No skin / eye contact – touching forbidden (for consumers and workers),

- use protective gloves and goggles

- avoid any dust / no use without LEV if dust arises (for workers in the formulation area)

- keep out of reach of children.

b) Product related measurements:

- formulate/ produce only dust free granulates,

- Warning: water may cause dangerous reactions in pipe cleaner packages

- packaging with child resistant closures only

c) Organisation measures: e.g.

- use only according to instructions

- regular control of compliance with instructions (workers)

- check of efficiency of the RMM (workers)

d) Technical measures:

e.g. transport in closed containers

e) Personal measures:

protective clothes, goggles, gloves

4. Control of environmental exposure:

a) Instructions:

- No emissions in water without neutralizing (only for industrial use)

- residuals in containers: clean / dilute with much water

c) Organisational measures:

- regular control of pH value: 8 – 9 (industrial use – formulating)

5. Waste: no special measures

6. Exposure estimation – exposure level (workers / consumer / environment):

No relevant exposure if RMM are effectively implemented / checked

For the internal use after the formulation in the formulator company no SDS will be elaborated. The company has to inform its own workers about the different OC and RMM for the formulated pipe cleaner by other tools like safety advices, training, instructions, etc. The difference in regard to the raw material relates to the not longer existing risks for flammability of Al, of explosive atmospheres of Al-dusts and oxidizing properties of NaNO3.

In the SDS for the professional cleaners as well as in the product information for consumers using the final product in small packages, the safety information would focus only on the prevention of skin and eye contact.

a) Instructions:

- No skin / eye contact – touching forbidden (for consumers and workers),

- use protective gloves and goggles

b) Product related measurements:

- Warning: water may cause dangerous reactions in pipe cleaner packages

c) Organisation measures: e.g.

- use only according to instructions

d) Personal measures:

protective clothes, goggles, gloves (specified like nitritbutylkautschuk according EN 374)

4. Control of environmental exposure:

a) Instructions:

- residuals in containers: clean / dilute with much water

Alternative:

Preparation of a customer information for a ready-to-use mixture

The consumer will not receive a safety data sheet. He will get all necessary product information by the labelling and the instructions on the label. These instructions would be at least:

- no skin / eye contact – touching forbidden, protect your eyes

- use protective gloves and goggles

- avoid any dust

- dilute residuals with very much water

- keep product dry – no water adding into the package

- in case of accident wash skin / eyes with much water, call doctor immediately if eye contact occured

9	Further interesting aspects of this case study

The use of existing knowledge on the mixtures is very helpful and in order to avoid new bureaucratic work it allows in addition that the addressees of the information (the SDS) will not be confronted with new or different advices. They had not to change their activities / documents except in the cases in which new information was created.

10	Information from the assessment of the mixture for the CSA of the substances

The use of the substance in a pipe cleaner is covered by the CSR of NaOH solid but not by the CSRs of the other raw materials (Al, NaNO3 and Na2CO3).

Annexes:

A new SDS has not been made, a CSR for NaOH is already available (prepared in part I of the REACH Practical Guide project).