REACH Practical Guide on Exposure Assessment and Communication in the Supply Chains

Examples to Part III: Mixtures under REACH

Case Study 3: Car Polish Product

This example consists of exposure scenarios for two substances used as raw materials for a mixture. In addition instructions for the formulators were developed on how to use the information from the ES of the raw materials and how to develop label instructions.

March 2010

Version: 1

Copyright

Copyright © of Cefic (AISBL) and VCI e.V., reproduction is authorised, except for commercial purposes, provided that the source is mentioned and acknowledged. Cefic and VCI claim no copyright on any official document, such as the REACH guidance documents (whose abstracts may be used in this publication) or information provided by the EU Institutions and ECHA.

Disclaimer

The information given in this REACH Guidance is intended for guidance only and whilst the information is provided in utmost good faith and has been based on the best information currently available, is to be relied upon at the user’s own risk. No representations or warranties are made with regards to its completeness or accuracy and no liability will be accepted for damages of any nature whatsoever resulting from the use of or reliance on the information. This does not apply if damage was caused intentionally or by gross negligence by Cefic or VCI or by parties assisting them.

Important note to the reader:

This case study has been prepared by the author within a VCI working group as part of the joint Cefic/VCI project to develop tools and guidances for industry – in respect of Chemical Safety Assessments, Chemical Safety Reports and Exposure Scenarios.

The REACH Practical Guide on Exposure Assessment and Communication in the Supply Chains developed in this project comprises several parts. An overview is given in the preface of Part I.

The structure and the contents of the REACH Practical Guide are described on the following web sites:

VCI: http://www.vci.de/default~cmd~shd~docnr~125022~lastDokNr~102474.htm

CEFIC: http://cefic.org/templates/shwPublications.asp?HID=750

All related documents can be downloaded from this site. In addition you find there information on related issues and actual developments.

These case studies have been prepared for REACH – related trainings purposes and educational purposes. It is not intended for the fulfilling of legal requirements. Case studies can be used as examples also by third parties if the source is cited.

It is mentioned in the description of the individual example if specific priorities have been set for the preparation of the example.

Author and contact person for this example:

Dr. Stefan Konietzny, Merck KGaA

## The background: Preparing safety data sheets and exposure scenarios for a mixture

Why we have chosen a car polish product as an example for a mixture?

The car polish product we have chosen is used as a consumer product. It is an interesting example as significant human exposure can be expected.

Which working steps are covered in the example?

In the example of the car polish product the following working steps have been performed:

- Identify substances of the mixture

- Check: Identification of lead substances useful?

- Collect required information for exposure scenario of the mixture / for the safety data sheet of the mixture

- Generate exposure scenarios for raw materials of the mixture (see Attachment 1 and Attachment 2)

Flow chart 1: Assessment of a mixture and generation of an extended safety data sheet

No eSDS was generated for the mixture as the mixture is a consumer product. However, label instructions were developed (see Attachment 3).

The example of the car polishing product illustrates:

- Guidance for formulators of products sold to the general public how to use information from exposure scenarios of raw materials

- Example for DU compliance check

- Examples for label instructions for consumers.

The working steps and the results

| 1 | Identify substances of the mixture and collect existing knowledge |
| --- | --- |

Table (1): Substances in the mixture (substance, content, R phrases)

| Name | CAS | [%] | Classification |
| --- | --- | --- | --- |
| Emulsimerck | -- | 1,5 | Xn R: 22-41 S: 26-39 |
| Carnaubawachs | 8015-86-9 | 2 | -- |
| Solvimerck | -- | 9 | Xn R: 10-65-66-67 S: 23-24-43-62 |
| Siliconoil 350 /10.000 | 63148-62-9 | 2 | -- |
| α-Al2O3 | 1344-28-1 | 3 | -- |
| C.I. Pigment Black | 1333-86-4 | 0,99 | -- |
| Water | 7732-18-5 | 81,51 | -- |

In the example of the car polish product, none of the substances is present below the relevant concentration limits.

| 2 | Document modifications of the determinants of exposure of the mixture |
| --- | --- |

The assessment of the car polish product is based on properties of the substances.

| 3 | Carry out classification and labelling (C&L) of the mixture |
| --- | --- |

The mixture does not have to be classified according to the criteria as laid down in Directive 1999/45/EC or to the criteria as laid down in Directive 67/548/EEC.

| 4 | Document differences between classification of the mixture and classification of the substances in the mixture. |
| --- | --- |

The mixture – in contrast to some of the substances – does not need to be classified.

| 5 | Check: Identification of lead substances useful? |
| --- | --- |

The lead substance identification approach was not used in this example.

| 6 | Identify lead substances and priority substances |
| --- | --- |

DPD+ has not been applied in the case of the car polish product.

No advanced evaluation was performed on the car polish product.

No specific operational conditions were considered for the car polish product.

No changes in the determinants of exposure occur for the car polish product.

| 7 | Collect required information for exposure scenario of the mixture / for the safety data sheet of the mixture |
| --- | --- |

The following information and/or tools have been used to identify adequate risk management measures (RMMs) and operational conditions (OCs):

- Exposure scenarios of raw materials

- Safety data sheets of raw materials

- Scaling

For the car polish product ES information on raw materials was generated for Solvimerck and Emulsimerck.

| 8 | Generate exposure scenario of mixture or integrate information into chapter 1 – 16 of the SDS |
| --- | --- |

8.1/8.2 	Compile and consolidate risk management measures and operational conditions

The car polish product is sold to the general public. Therefore, no extended SDS was generated.

However, instructions for the formulators were developed on how to use the information from the ES of the raw materials and how to develop label instructions (see Attachment 3).

8.3 Check whether identified risk management measures for lead substances cover other substances in the mixture

See Attachment 3.

8.4	Decide on scaling guidance and – if yes – develop this guidance

See Attachment 3.

8.5	Decide in which form the information should be included in the safety data sheet     of the mixture

As mentioned above, in the case of the car polish product no extended safety data sheet has been prepared, because the product is sold to the general public.

9	Further interesting aspects of this case study

As already mentioned above, instructions for the formulators were developed on how to use the information from the ES of the raw materials and how to develop label instructions (see Attachment 3).

Attachments

Attachment 1: Exposure scenario for raw material 1 (solvent)

Attachment 2: Exposure scenario for raw material 2 (emulsifier)

Attachment 3: Information for the formulator (explanation of ES, guidance for compliance check and scaling, consolidation of exposure scenarios 1 and 2, label instructions).