REACH Practical Guide on Exposure Assessment and Communication in the Supply Chains

Examples to Part III: Mixtures under REACH

Case Study 1: 2 component adhesive

This example illustrates the use of the DPD+ approach to generate an exposure scenario for a mixture with focus on aspects related to the environment.

March 2010

Version: 1

Copyright

Copyright © of Cefic (AISBL) and VCI e.V., reproduction is authorised, except for commercial purposes, provided that the source is mentioned and acknowledged. Cefic and VCI claim no copyright on any official document, such as the REACH guidance documents (whose abstracts may be used in this publication) or information provided by the EU Institutions and ECHA.

Disclaimer

The information given in this REACH Guidance is intended for guidance only and whilst the information is provided in utmost good faith and has been based on the best information currently available, is to be relied upon at the user’s own risk. No representations or warranties are made with regards to its completeness or accuracy and no liability will be accepted for damages of any nature whatsoever resulting from the use of or reliance on the information. This does not apply if damage was caused intentionally or by gross negligence by Cefic or VCI or by parties assisting them.

Important note to the reader:

This case study has been prepared by the author within a VCI working group as part of the joint Cefic/VCI project to develop tools and guidances for industry – in respect of Chemical Safety Assessments, Chemical Safety Reports and Exposure Scenarios.

The REACH Practical Guide on Exposure Assessment and Communication in the Supply Chains developed in this project comprises several parts. An overview is given in the preface of Part I.

The structure and the contents of the REACH Practical Guide are described on the following web sites:

VCI: http://www.vci.de/default~cmd~shd~docnr~125022~lastDokNr~102474.htm

CEFIC: http://cefic.org/templates/shwPublications.asp?HID=750

All related documents can be downloaded from this site. In addition you find there information on related issues and actual developments.

These case studies have been prepared for REACH – related trainings purposes and educational purposes. It is not intended for the fulfilling of legal requirements. Case studies can be used as examples also by third parties if the source is cited.

It is mentioned in the description of the individual example if specific priorities have been set for the preparation of the example.

Author and contact person for this example:

Dr. Christian Bögi, BASF SE

## The background: Preparing safety data sheets and exposure scenarios for a mixture

The following example illustrates steps in the process of assessing a mixture and preparing the related exposure scenario and the extended safety data sheet for the mixture.

The main steps in this process are shown in flowchart 1. It includes the use of existing knowledge, the requirements for the classification and labelling of a mixture and also the new obligations under REACH. Flowchart 1 shows the whole process from the identification of the substance profile of the mixture and its hazard assessment until the preparation of the safety data sheet of the mixture.

A general description of the working steps is given in Part III, chapter 6.1 of the REACH Practical Guide.

Note: It has not been the aim of these case studies to prepare complete chemical safety assessments for the substances in the mixtures. Examples for chemical safety assessments and the related chemical safety reports can be found in Part V of the REACH Practical Guide.

Why we have chosen a 2K-adhesive as an example for a mixture?

We aimed to work on a fictitious, but realistic example with high practical relevance. In order to be able to apply DPD+, we have chosen a product which does not contain any compounds triggering advanced evaluation.

Which working steps are covered in the example?

In the example of the 2K-adhesive the working following steps have been performed:

- Identify substances of the mixture

- Check: Identification of lead substances useful?

- Identify lead substances

- Collect required information for exposure scenario of the mixture / for the safety data sheet of the mixture

- Generate exposure scenario for the mixture

Flow chart 1: Assessment of a mixture and generation of an extended safety data sheet

The working steps and the results

| 1 | Identify substances of the mixture and collect existing knowledge |
| --- | --- |

Table 1 shows the substances in the mixture.

Table (1): Substances in the mixture (substance, content, R phrases)

| Name | CAS | [%] | Classification |
| --- | --- | --- | --- |
| Bisphenol-A-epichlorhydrin resins | 25068-38-6 | 60 | Xi; R36/38 R43 N; R51-53 |
| Xylene | 1330-20-7 | 10 | R10 Xn; R20/21 Xi; R38 |
| Isophorone diamine | 2855-13-2 | 18 | Xn; R21/22 C; R34 R43 R52-53 |
| Benzyl alcohol | 100-51-6 | 10 | Xn; R20/22 |
| Ethanol | 64-17-5 | 2 | F; R11 |

In the example of the 2K-adhesive none of the substances has been below the concentration limits.

| 2 | Document modifications of the determinants of exposure of the mixture |
| --- | --- |

For many mixtures, formulators make an expert judgement on characteristics of the mixture (Physicochemical properties, human health properties, environmental properties). It is based on their knowledge of the mixture.

The assessment of the 2K-adhesive is based on

- properties of the substances

| 3 | Carry out classification and labelling (C&L) of the mixture |
| --- | --- |

The mixture is classified as:

C; R 34, R 43

N; R 51-53

| 4 | Document differences between classification of the mixture and classification of the substances in the mixture. |
| --- | --- |

Such differences indicate that information from exposure scenarios of raw materials might need modification before they can be used for the exposure scenario of the mixture.

The mixture – in contrast to some of the substances – does not need to be classified with R10, F, R11, or Xi, R36/38 or Xi, R38 or Xn, R20-21 or Xn, R20-22 or Xn, R21-22, respectively.

| 5 | Check: Identification of lead substances useful? |
| --- | --- |

The lead substance approach is useful for substance-rich mixtures if the hazard assessment of the mixtures is based on data of the substances. If the hazard assessment is testing of the mixture or on expert knowledge, identification of lead substances is not required.

As no information on the mixture was available, identifiying the lead substance(s) appeared useful.

| 6 | Identify lead substances and priority substances |
| --- | --- |

A description of the methodology to identify lead substances is given in Part III, chapter 7.2 of the REACH Practical Guide.

Lead substances were identified using DPD+ as described in the REACH Pracitical Guide.

The input information is shown in the following table 2

Table (2)  Input information for DPD+

(see register “Input”)

The results of the application of DPD+ are shown in the following table 3.

Table (3)  Result of application of DPD+

(see register “Output”)

Lead compounds of "2K-adhesive" identified by DPD+ were:

Inhalation 	Xylene

Dermal 	BPA-resin

Eyes  	BPA-resin

Ingestion  	Isophorone diamine

Aquatic Environment	BPA-resin

Exposure pathway “ingestion” was considered not relevant as no consumer exposure is foreseen for 2K-adhesive. During professional use, an oral exposure is not expected.

Experiences with DPD+:

DPD+ worked well within its applicability.

Generally, DPD+ uses information on hazard only to define lead substances that are then used to assess the risk of the mixture. That may have implications. Expert knowledge on the product needs to be considered.

Advanced evaluation: Does your mixture contain priority substances which are not covered by the DPD+ method (e.g. CMR 1 + 2 substances, PBT/vPvB substances) in relevant concentrations which require an advanced evaluation of the mixture?

No advanced evaluation was performed on the 2K-adhesive.

Specific operational conditions: Do you have to consider specific operational conditions which require an adjustment of the risk management measures?

No specific operational conditions were considered for the 2K-adhesive.

Changes in exposure: Do you have to consider specific changes in the determinants of exposure for your mixture (e.g. differences in the bioavailability of the substances in the mixture) which require an adjustment of the risk management measures?

No changes in the determinants of exposure occur for the 2K-adhesive.

| 7 | Collect required information for exposure scenario of the mixture / for the safety data sheet of the mixture |
| --- | --- |

Information on risk management measures and operational conditions for mixtures can be taken from several sources. If no exposure scenarios are available for the lead substances of the mixture, starting point could be safety data sheets of the substances. In addition, sector-specific information on the use of the substances and mixtures could used. RMMS and OCs can be determined by using exposure assessment tools as COSSH essential / EMKG, see Part III, chapter 6.2  of the REACH Practical Guide.

The following information and/or tools have been used to identify adequate risk management measures (RMMs) and operational conditions (OCs):

- Exposure scenarios of raw materials

- Safety data sheets of raw materials

- Scaling

As the “2K-adhesive” was a fictitious example, no complete SDSs and ES information on raw materials were available. In order to focus on the target of the project, only those parts of substance information necessary to built the ES for the mixture were generated.

Exposure scenarios of raw materials

For the lead substance identified for the aquatic environment by DPD+, BPA-resin, an exposure estimation was carried out using ECETOC TRA. The following information was assumed to be part of an exposure scenario, which a supplier would communicate to his downstream user via the annex of an eSDS:

C: concentration of the substance in the product:	1

f water: emission factor:		0.01

f abatement: efficiency of an abatement:		0

f STP: removal of substance in STP:		0.15

T emission: duration of emission [d/year]:		365

CAPACITY: water treated in the STP [m3/d]:		18000

DILUTION: dilution factor in the receiving water body:	10

RCR:		1

M safe [kg/d]:		2.23

For the inhalation lead substance identified by DPD+, Xylene, estimated exposure values were calculated using ECETOC TRA worker v2.0. The information below was assumed to be part of an exposure scenario included in the annex of the suppliers´ eSDS:

Application domain:	professional

f Dur: Duration of activity:	> 4h  / f = 1

Scaling: 	1-4h / f = 0,6

15min – 1h / f = 0,2

< 15min / f = 0,1

f LEV: Ventilation	Indoor without LEV / f = 1

Scaling: 	Indoor with LEV / f = 0,2

Indoor with LEV / depend. on LEV eff.

Outdoor / f = 0,7

f RP: Respiratory protection	Yes (90%) / f = 0,1

Scaling: 	Yes / f = 0,05

dependent on RP efficiency

f Prep: Preparation	No / f = 1

Scaling: dependent on concentration of substance

Expo inhal, ES	215 mg/m³

RCR	0.49

For the dermal lead substance and the eye lead substance a qualitative assessment was carried out. The following information was assumed to be part of the exposure scenario communication for downstream users via the annex of the suppliers eSDS:

Data on Mixture: No

Technical conditions –  Control of dispersion from source:

- Use of suitable tools (e.g. rakes, scrapes)

Organisational measures – Limit release, dispersion and exposure:

- Check gloves for damage before the start of the activity,

- Cleaning of gloves under running water before taking them off

Conditions related to personal protection, hygiene, and health evaluation:

- Wearing of tightly fitting safety goggles

- Wearing of suitable chemical resistent gloves

- Avoid skin and eye contact

| 8 | Generate exposure scenario of mixture or integrate information into chapter 1 – 16 of the SDS |
| --- | --- |

8.1/8.2 	Compile and consolidate risk management measures and operational conditions

In order to generate the exposure scenario for the mixture, the relevant pieces of information from the exposure scenarios of the identified lead substances have been taken and combined to the exposure scenario of the mixture.

The following figure shows which elements from the exposure scenarios of the raw materials have been combined to compile the exposure scenario of the mixture.

The environmental part from the exposure scenario of the lead substance BPA-DGE is documented in the following file:

In the case of the 2K-adhesive, the following parameters have been different in the mixture from the assumptions in the exposure scenarios of the raw materials:

- The concentration of the lead substance in the mixture;

- The emission factor to water;

- The use of respiratory protection;

- The use of a local exhaust ventilation.

Scaling guidance in the exposure scenarios of the raw materials have been used to adapt the information from exposure scenarios of substances to the specific conditions of use in the mixture – see section 8.4.

In the case of the 2K-adhesive, the use of all substances in the mixture has been covered by the exposure scenarios of the substances received from the supplier.

8.3 Check whether identified risk management measures for lead substances cover other substances in the mixture

Priority substances require most stringent risk management measures. However, if these measures are substance-specific (e.g. a precipitation step), it is possible that they do not control adequately further other substances of the mixture with different physicochemical properties. Therefore it is recommended to apply control steps to ensure that the recommended risk management measures are really adequate for the mixture. These steps are shown in Flow Chart 3 in Part III, chapter 7.3  of the REACH Practical Guide.

The risk management measures for the lead substances of the 2K-adhesive cover also the other substances in the mixture.

8.4	Decide on scaling guidance and – if yes – develop this guidance

Scaling can broaden the range of applications which are covered by an exposure scenario. Scaling means the use of simple equations in the exposure scenario by which the downstream user can demonstrate that he operates within the conditions of the exposure scenario provided by the registrant. It is described in Part III, chapter 7.7 of the REACH Practical Guide.

Environment:

Assumptions made for the downstream users situation:

- Concentration of the lead substance in the mixture: 	0.6 (60%)

- f water: emission factor 		0.002
(based on measurements)

To derive the data to be included into the environmental part of the exposure scenario for the mixture – based on the information available for the relevant lead substance and on the assumptions for the DUs situation stated above – the following equation was used:

0.01

0.002

1

0.6

2.23

actual

Msafe

1

1



x

x

x

18.55

actual

Msafe



Inhalation:

Assumptions made for the downstream user situation:

Concentration of lead substance in the mixture (f Prep):	10% (f Prep: 0.1)

Use of respiratory protection (f RP):		No (f RP: 1)

Local exhaust ventilation (f LEV):		Yes (f LEV: 0.2)

To derive the data relevant for inhalation exposure to be included into the exposure scenario for the mixture – based on the information available for the relevant lead substance and on the assumptions for the DUs situation stated above – the following equations were used:

Expoinhal, actual=Expoinhal, ES×fLEV, actualfLEV, ES×fDur, actualfDur, ES×fRP,acutalfRP,ES×fPrep,actualfPrep,ES

Expoinhal, actual=    215     ×  0,2       1,0 × 1,0 1,0× 1,0    0,1 ×   0,1     1,0

Expoinhal, actual=    215     ×  0.2    1.0 × 1.0 1.0× 1.0    0.1 ×   0.1     1.0

Expo inhal, acutal  = 43 mg/m³

RCR inhal, acutal  = 0.1

Dermal and eye:

As a qualitative assessment was done, no scaling is possible. For the exposure scenario of the mixture the conditions communicated in the suppliers´ eSDS has to be adopted.

8.5	Decide in which form the information should be included in the safety data sheet     of the mixture

Downstream users have to include in their own safety data sheets information which they have received from their suppliers (REACH Art. 31.7, see chapter 7.2). They can do this in several ways. They can forward exposure scenarios, they can consolidate information (into an exposure scenario for the mixture) or they can integrate information into chapter 1 – 16  of the safety data sheet (see chapter 7.6 of the REACH Practical Guide).

If the information provided by the supplier is sufficient and suitable to build an appropriate exposure scenario for the mixture, this is the choice to make. A discrete exposure scenario allows for the provision and further communication of all relevant information tailored to the properties and use of the mixture, all concentrated in one part of the eSDS.

In the case of the 2K-adhesive, inclusion of information has been done in the following way:

- Consolidation of information into an exposure scenario for the mixture which is annexed to the extended safety data sheet.

8.5	Document the result: exposure scenario and / or extended safety data sheet of the mixture

In the following file the exposure scenario of the 2K-adhesive  is  documented (environmental part).