REACH Practical Guide on Exposure Assessment and Communication in the Supply Chains

Examples to Part III: Mixtures under REACH

Case Study 6: Industrial Cleaner

This example illustrates the assessment of a complex ionic mixture with reactions taking place in the mixture.

March 2010

Version: 1

Copyright

Copyright © of Cefic (AISBL) and VCI e.V., reproduction is authorised, except for commercial purposes, provided that the source is mentioned and acknowledged. Cefic and VCI claim no copyright on any official document, such as the REACH guidance documents (whose abstracts may be used in this publication) or information provided by the EU Institutions and ECHA.

Disclaimer

The information given in this REACH Guidance is intended for guidance only and whilst the information is provided in utmost good faith and has been based on the best information currently available, is to be relied upon at the user’s own risk. No representations or warranties are made with regards to its completeness or accuracy and no liability will be accepted for damages of any nature whatsoever resulting from the use of or reliance on the information. This does not apply if damage was caused intentionally or by gross negligence by Cefic or VCI or by parties assisting them.

Important note to the reader:

This case study has been prepared by the author within a VCI working group as part of the joint Cefic/VCI project to develop tools and guidances for industry – in respect of Chemical Safety Assessments, Chemical Safety Reports and Exposure Scenarios.

The REACH Practical Guide on Exposure Assessment and Communication in the Supply Chains developed in this project comprises several parts. An overview is given in the preface of Part I.

The structure and the contents of the REACH Practical Guide are described on the following web sites:

VCI: http://www.vci.de/default~cmd~shd~docnr~125022~lastDokNr~102474.htm

CEFIC: http://cefic.org/templates/shwPublications.asp?HID=750

All related documents can be downloaded from this site. In addition you find there information on related issues and actual developments.

These case studies have been prepared for REACH – related trainings purposes and educational purposes. It is not intended for the fulfilling of legal requirements. Case studies can be used as examples also by third parties if the source is cited.

It is mentioned in the description of the individual example if specific priorities have been set for the preparation of the example.

Author and contact person for this example:

Dr. Sybille Höhmann, Henkel AG & Co. KGaA

## The background: Preparing safety data sheets and exposure scenarios for a mixture

The following example illustrates steps in the process of assessing a mixture and preparing the related exposure scenario and the extended safety data sheet for the mixture.

The main steps in this process are shown in flowchart 1. It includes the use of existing knowledge, the requirements for the classification and labelling of a mixture and also the new obligations under REACH. Flowchart 1 shows the whole process from the identification of the substance profile of the mixture and its hazard assessment until the preparation of the safety data sheet of the mixture.

A general description of the working steps is given in Part III, chapter 6.1 of the REACH Practical Guide.

Note: It has not been the aim of these case studies to prepare complete chemical safety assessments for the substances in the mixtures. Examples for chemical safety assessments and the related chemical safety reports can be found in Part V of the REACH Practical Guide.

Why we have chosen an industrial cleaner as an example for a mixture?

An industrial cleaner has been selected as an example because it is an ionic mixture with  reactions taking place. In addition it contains a chemical substance which will be classified as reprotoxic according to the 1st ATP of CLP.

Which working steps are covered in the example?

In the example of the industrial cleaner the following working steps have been performed:

- Identify substances of the mixture and collect existing knowledge

- Carry out classification and labelling (C&L) of the mixture

- Check: Identification of lead substances useful?

- Identify lead substances

- Collect required information for exposure scenario of the mixture / for the safety data sheet of the mixture

- Generate exposure scenario for mixture

- Flow chart 1: Assessment of a mixture and generation of an extended safety data sheet

In addition to the task indicated above, the example of the industrial cleaner illustrates

the issue of an expert judgement with regard to the CMR substance.

The working steps and the results

| 1 | Identify substances of the mixture and collect existing knowledge |
| --- | --- |

Table (1): Raw materials used for the mixture

| Raw material | Content |
| --- | --- |
| Sodium tetraborate x 5 H2O | 10,00% |
| Sodium metasilicate 56 % | 8,93% |
| Phosphoric acid 75 % | 7,97% |
| Sodium hydroxide (50 %) | 24,64% |
| Nonionic Surfactants | 5,00% |
| Triethanolamine (99%) | 1,01% |
| Anionic Surfactants | 2,00% |
| Sodiumcumolsulfonate (93%) | 7,53% |
| Water | 32,92% |

Table (2): Dangerous substances in the mixture above the concentration limits

including chemical reactions:

| Chemical Substance | CAS-No | R-Phrases | % |
| --- | --- | --- | --- |
| Sodium tetraborate | 1330-43-4 | (R60, R61 according to 1.ATP CLP) | 7,00% |
| Sodium metasilicate | 6834-92-0 | R34,R37 | 5,00% |
| Trisodium phosphate | 7601-54-9 | R36, R37, R38 | 10,00% |
| Sodium hydroxide | 1310-73-2 | R35 | 5,00% |

Some of the substances in the mixture might be not relevant for the assessment because they are (1) below concentration limits of DPD / REACH (0.1%, 1% or substance specific limit values, see REACH Art. 14.2) or they are (2) non dangerous / non PBT / non vPvB. Application of these criteria can reduce the number of substances which is further considered.

| 2 | Document modifications of the determinants of exposure of the mixture |
| --- | --- |

For many mixtures, formulators make an expert judgement on characteristics of the mixture (Physicochemical properties, human health properties, environmental properties). It is based on their knowledge of the mixture.

The assessment of the industrial cleaner is based on

- properties of the substances

- knowledge about the occurring reactions

A reaction takes place between Sodium hydroxide and Phosphoric acid.

| 3 | Carry out classification and labelling (C&L) of the mixture |
| --- | --- |

The product is currently classified as R34.

| 4 | Document differences between classification of the mixture and classification of the substances in the mixture. |
| --- | --- |

Such differences indicate that information from exposure scenarios of raw materials might need modification before they can be used for the exposure scenario of the mixture.

Different from the classification of the mixture (R34), the substances used as raw materials are classified as R34, R35, R36, R37, R 38, R60, R61 (see table 2).

| 5 | Check: Identification of lead substances useful? |
| --- | --- |

The lead substance approach is useful for substance-rich mixtures if the hazard assessment of the mixtures is based on data of the substances. If the hazard assessment is testing of the mixture or on expert knowledge (e.g. bridging from other mixtures), identification of lead substances is not required.

It is useful to determine leading substances for the different paths in order to reduce information you have to include in the extended SDS.

| 6 | Identify lead substances and priority substances |
| --- | --- |

A description of the methodology to identify lead substances is given in Part III, chapter 7.2 of the REACH Practical Guide.

Sodium hydroxide is the leading substance for all paths.

Experiences with DPD+:

The DPD+-method reduces the amount of relevant substances to only one substance for the different exposure paths.

Advanced evaluation: Does your mixture contain priority substances which are not covered by the DPD+ method (e.g. CMR 1 + 2 substances, PBT/vPvB substances) in relevant concentrations which require an advanced evaluation of the mixture?

The mixture contains sodium tetraborate (R60/61 according to 1. ATP CLP).

An expert judgement is in this case automatically triggered:

The German OEL of 0.5 mg/m3 (TRGS 900) is chosen as DNEL(worker) because the underlying risk assessment already takes into account potential reproductive effects.

The exposure estimate calculated by the ECETOC TRA program is 0,167 mg/m3 which is lower than the above mentioned DNEL/OEL.

In consequence, no adjustment of the risk management measures that are already in place to protect against corrosive/irritating properties is necessary. The use of protective gloves is required in chapter 8 of the SDS.

The extended SDS in this example refers to the legal requirements of the 29.ATP.

Specific operational conditions: Do you have to consider specific operational conditions which require an adjustment of the risk management measures?

Not applicable.

Changes in exposure: Do you have to consider specific changes in the determinants of exposure for your mixture (e.g. differences in the bioavailability of the substances in the mixture) which require an adjustment of the risk management measures?

Not applicable.

| 7 | Collect required information for exposure scenario of the mixture / for the safety data sheet of the mixture |
| --- | --- |

Information on risk management measures and operational conditions for mixtures can be taken from several sources. If no exposure scenarios are available for the lead substances of the mixture, starting point could be safety data sheets of the substances. In addition, sector-specific information on the use of the substances and mixtures could used. RMMS and OCs can be determined by using exposure assessment tools as COSSH essential / EMKG, see Part III, chapter 6.2  of the REACH Practical Guide.

No exposure scenarios for the substances of the mixture have so far been provided by the manufacturers.

The following information and/or tools have been used to identify adequate risk management measures (RMMs) and operational conditions (OCs):

•  Safety data sheets of raw materials

•  Sector-specific information on RMMs and OCs

•  Generic Exposure Scenarios by ECETOC TRA:

Exposure values for all categories (PROCs) considered by AISE for the product type under consideration have been estimated for the lead substances (sodium hydroxide and disodium tetraborate) and compared with the respective DNELs (NOAELs) derived from the scientific literature.

The ECETOC TRA exposure scenario was mainly chosen to cover the formal aspects of an extended SDS and to demonstrate the principal workability of the approach. However, ECETOC TRA deals only with dusts and vapours whereas the lead substances are "solids-in-water" and even at elevated temperatures aqueous vapours do not contain appreciable amounts of sodium hydroxide. For this reason, it is a conservative approach to treat solids dissolved in aqueous solutions as ingredients with low vapour pressures. Therefore, in cases like this it should be possible to alternatively assess mixtures on the basis of the existing information and expert knowledge without using the calculated ECETOC TRA exposure concentrations.

| 8 | Generate exposure scenario of mixture or integrate information into chapter 1 – 16 of the SDS |
| --- | --- |

8.1/8.2 	Compile and consolidate risk management measures and operational conditions

In order to generate the exposure scenario for the mixture, the relevant pieces of information from the exposure scenarios of the identified lead substances have been taken and combined to the exposure scenario of the mixture.

It might be that information from exposure scenarios of substances has to be adapted to the specific conditions of use in the mixture. See Part III, chapter 6.3 of the Practical Guide for details.

In the case of the industrial cleaner exposure scenarios from the suppliers are currently not available. Therefore, calculations are based on generic exposure models (ECETOC TRA). It is not expected that a report to ECHA is necessary.

8.3 Check whether identified risk management measures for lead substances cover other substances in the mixture

Priority substances require most stringent risk management measures. However, if these measures are substance-specific (e.g. a precipitation step), it is possible that they do not control adequately further other substances of the mixture with different physicochemical properties. Therefore it is recommended to apply control steps to ensure that the recommended risk management measures are really adequate for the mixture. These steps are shown in Flow Chart 3 in Part III, chapter 7.3 of the REACH Practical Guide.

For the industrial cleaner, the following aspects require an adjustment of the risk management measures:

•	RMMs/OCs for lead substances cover all other substances of the mixture

8.4	Decide on scaling guidance and – if yes – develop this guidance

Scaling can broaden the range of applications which are covered by an exposure scenario. Scaling means the use of simple equations in the exposure scenario by which the downstream user can demonstrate that he operates within the conditions of the exposure scenario provided by the registrant. It is described in Part III, chapter 7.7 of the REACH Practical Guide.

Scaling has not been considered in this example.

8.5	Decide in which form the information should be included in the safety data sheet     of the mixture

Downstream users have to include in their own safety data sheets information which they have received from their suppliers (REACH Art. 31.7, see chapter 7.2). They can do this in several ways. They can forward exposure scenarios, they can consolidate information (into an exposure scenario for the mixture) or they can integrate information into chapter 1 – 16  of the safety data sheet (see Part III, chapter 6 of the REACH Practical Guide).

In the case of the industrial cleaner inclusion of information has been done in the following way:

- Consolidated information into an exposure scenario for the mixture which is annexed to the extended safety data sheet;

- Integration of information into chapter 1 – 16 of the extended safety data sheet;

Forwarding exposure scenarios of substances which are used as raw materials for the mixture has a limited benefit for the downstream user due to the supposed information cascade effect in the supply chain.

8.6	Document the result: exposure scenario and / or extended safety data sheet of the mixture

The e-SDS of the industrial cleaner is documented in the following file: