REACH Practical Guide on Exposure Assessment and Communication in the Supply Chains

Examples to Part III: Mixtures under REACH

Case Study 4: Engine Oil Formulation

This example illustrates the use of the DPD+ approach to identify lead substances for a mixture and the integration of the information into the main body of the extended safety data sheet.

March 2010

Version: 1

Copyright

Copyright © of Cefic (AISBL) and VCI e.V., reproduction is authorised, except for commercial purposes, provided that the source is mentioned and acknowledged. Cefic and VCI claim no copyright on any official document, such as the REACH guidance documents (whose abstracts may be used in this publication) or information provided by the EU Institutions and ECHA.

Disclaimer

The information given in this REACH Guidance is intended for guidance only and whilst the information is provided in utmost good faith and has been based on the best information currently available, is to be relied upon at the user’s own risk. No representations or warranties are made with regards to its completeness or accuracy and no liability will be accepted for damages of any nature whatsoever resulting from the use of or reliance on the information. This does not apply if damage was caused intentionally or by gross negligence by Cefic or VCI or by parties assisting them.

Important note to the reader:

This case study has been prepared by the author within a VCI working group as part of the joint Cefic/VCI project to develop tools and guidances for industry – in respect of Chemical Safety Assessments, Chemical Safety Reports and Exposure Scenarios.

The REACH Practical Guide on Exposure Assessment and Communication in the Supply Chains developed in this project comprises several parts. An overview is given in the preface of Part I.

The structure and the contents of the REACH Practical Guide are described on the following web sites:

VCI: http://www.vci.de/default~cmd~shd~docnr~125022~lastDokNr~102474.htm

CEFIC: http://cefic.org/templates/shwPublications.asp?HID=750

All related documents can be downloaded from this site. In addition you find there information on related issues and actual developments.

These case studies have been prepared for REACH – related trainings purposes and educational purposes. It is not intended for the fulfilling of legal requirements. Case studies can be used as examples also by third parties if the source is cited.

It is mentioned in the description of the individual example if specific priorities have been set for the preparation of the example.

Author and contact person for this example:

Sabine Hausmann, Fuchs Europe Schmierstoffe GmbH

## The background: Preparing safety data sheets and exposure scenarios for a mixture

The following example illustrates steps in the process of assessing a mixture and preparing the related exposure scenario and the extended safety data sheet for the mixture.

The main steps in this process are shown in flowchart 1. It includes the use of existing knowledge, the requirements for the classification and labelling of a mixture and also the new obligations under REACH. Flowchart 1 shows the whole process from the identification of the substance profile of the mixture and its hazard assessment until the preparation of the safety data sheet of the mixture.

A general description of the working steps is given in chapter 7.3 of the REACH Practical Guide on Exposure Assessment and Communication in the Supply Chains.

Note: It has not been the aim of these case studies to prepare complete chemical safety assessments for the substances in the mixtures. Examples for chemical safety assessments and the related chemical safety reports can be found in Part V of the REACH Practical Guide.

Why we have chosen an Engine Oil Formulation as an example for a mixture?

An Engine Oil Formulation has been selected as example because

- it is a typical example for a mixture which is manufactured by mixing various raw materials: a substance (Mineral Oil) and another mixture (Additive Package).

- It covers industrial, professional and consumer use

Note: Risk management measures for the environment have not been addressed in detail in this case study.

Which working steps are covered in the example?

In the example of the engine oil the following working steps have been performed:

- Identify substances of the mixture and collect existing knowledge

- Document modifications of the determinants of exposure of the mixture

- Carry out classification and labelling (C&L) of the mixture

- Document differences between classification of the mixture and classification of the substances in the mixture.

- Check, whether DPD+ Methodology can be applied and identification of lead substances is useful

- Identify lead substances

- Collect required information for exposure scenario of the mixture / for the safety data sheet of the mixture

- Generate exposure scenario for mixture or integrate information into chapter 1 – 16 of the SDS

Flow chart 1: Assessment of a mixture and generation of an extended safety data sheet

The working steps and the results

| 1 | Identify substances of the mixture and collect existing knowledge |
| --- | --- |

For the evaluation of the engine oil detailed information on the constituents, their concentration in the formulated “additive package” and their physico-chemical, toxicological and eco-toxicological properties had to be collected from the supplier of the additive package.

Table (1): Raw materials used for the mixture (engine oil)

| Substance Name | Concentration | Classification & Labelling |
| --- | --- | --- |
| Mineral Oil | 81.25 % | - |
| Additive Package | 18.75 % | Xi R 41, R52/53 |

| Additive Package | Concentration |
| --- | --- |
| Hindered Phenol | 26.7 % |
| Zinc dialkyl dithiophosphate (ZDDP) | 15.0 % |
| Calcium Sulphonate | 4.8 % |
| Dispersant | 18.1 % |
| Viscosity Modifier | 17.6 % |
| Mineral Oil | 17.8 % |

Table (2): Substances in the mixture (substance, content, R phrases)

| Engine Oil Formulation | Concentration | R-Phrase(s) | Concentration Limit |
| --- | --- | --- | --- |
| Hindered Phenol | 5.0 % | R 53 | 25.0 % |
| Zinc dialkyl dithiophosphate (ZDDP) | 2.8 % | R 38 R 41 R 51/53 | 20.0 % 10.0 % 2.5 % |
| Calcium Sulphonate | 0.9 % | R 53 | 25.0 % |
| Dispersant | 3.4 % | - | Not relevant |
| Viscosity Modifier | 3.3 % | - | Not relevant |
| Mineral Oil | 84.6 % | - | Not relevant |

Some of the substances in the mixture might be not relevant for the assessment because they are (1) below concentration limits of DPD / REACH (0.1%, 1% or substance specific limit values, see REACH Art. 14.2) or they are (2) non dangerous / non PBT / non vPvB. Application of these criteria can reduce the number of substances which is further considered.

| 2 | Document modifications of the determinants of exposure of the mixture |
| --- | --- |

For many mixtures, formulators make an expert judgement on characteristics of the mixture (Physicochemical properties, human health properties, environmental properties). It is based on their knowledge of the mixture.

The hazard assessment of the engine oil is based on the hazard properties of the substances (see Table 2). The engine oil was neither tested as such nor was information with regards to potential interactions between constituents known.

| 3 | Carry out classification and labelling (C&L) of the mixture |
| --- | --- |

| Engine Oil Formulation | Concen-tration | R-Phrase(s) | Concen-tration Limit | LSI | Engine Oil Final C & L |
| --- | --- | --- | --- | --- | --- |
| Hindered Phenol | 5.0 % | R 53 | 25.0 % | 0.20 | - |
| Zinc dialkyl dithiophosphate (ZDDP) | 2.8 % | R 38 R 41 R51/53 | 20.0 % 10.0 % 2.5 % | 0.14 0.28 1.12 | - - R 52/53 |
| Calcium Sulphonate | 0.9 % | R 53 | 25.0 % | 0.04 | - |
| Dispersant | 3.4 % | - | Not relevant | - | - |
| Viscosity Modifier | 3.3 % | - | Not relevant | - | - |
| Base Oil | 84.6 % | - | Not relevant | - | - |

| 4 | Document differences between classification of the mixture and classification of the substances in the mixture. |
| --- | --- |

Such differences indicate that information from exposure scenarios of raw materials might need modification before they can be used for the exposure scenario of the mixture.

The additive package is classified as irritant (Xi) with risk of serious damage to eyes (R 41) and as harmful to aquatic organisms, may cause long-term adverse effects in the aquatic environment (R 52/53) due to the concentration of Zinc dialkyl dithiophosphate (ZDDP) present.

The engine oil is only classified as harmful to aquatic organisms, may cause long-term adverse effects in the aquatic environment (R 52/53). The reduction in classification is due to the dilution effect of mineral oil which is not classified as a skin or eye irritant. Thus, the lead substance for skin and eye irritation (i.e., ZDDP) is below the LSI threshold and thus the finished engine oil is no longer classified as a skin or eye irritant.

| 5 | Check: Identification of lead substances useful? |
| --- | --- |

The lead substance approach is useful for mixtures where the hazard assessment of the mixture is based on data of the individual substances (raw materials). If the hazard assessment is based on testing of the mixture or on expert knowledge (e.g. bridging from similar mixtures), identification of lead substances is not required.

None of the lubricant additive components (raw materials) are classified as:

- carcinogens, mutagens or substances toxic to reproduction (CMR’s)

- respiratory sensitizers

- PBT or vPvB substances

Therefore no advanced evaluation is required: DPD+ Methodology can be applied and the identification of Lead Substance(s) is useful

| 6 | Identify lead substances and priority substances, as required |
| --- | --- |

A description of the methodology to identify lead substances is given in Part III, chapter 7.2 of the REACH Practical Guide.

Lead substances have been identified according to DPD+ Methodology as described in section 7 of the REACH Practical Guide. Details can be found in the Table in the Annex 1 to this case study.

Zinc dialkyl dithiophosphate has been identified as Lead Substance for aquatic environment.

Experiences with DPD+:

Application of DPD+ Methodology was straight forward. Additionally it helps to ensure consistency of all parts of the Safety Data Sheet with the required exposure scenarios.

Advanced evaluation: Does your mixture contain priority substances which are not covered by the DPD+ method (e.g. CMR 1 + 2 substances, PBT/vPvB substances) in relevant concentrations which require an advanced evaluation of the mixture?

The mixture does not contain any substances which require an advanced evaluation of the mixture.

Specific operational conditions: Do you have to consider specific operational conditions which require an adjustment of the risk management measures?

Not applicable

Changes in exposure: Do you have to consider specific changes in the determinants of exposure for your mixture (e.g. differences in the bioavailability of the substances in the mixture) which require an adjustment of the risk management measures?

Not applicable.

| 7 | Collect required information for exposure scenario of the mixture / for the safety data sheet of the mixture |
| --- | --- |

Information on risk management measures and operational conditions for mixtures can be taken from several sources. If no exposure scenarios are available for the lead substances of the mixture, starting point could be safety data sheets of the substances. In addition, sector-specific information on the use of the substances and mixtures could be used. RMMs and OCs can be determined by using exposure assessment tools as COSHH essential / EMKG, see Part III, chapter 6.2  of the REACH Practical Guide.

The following information and/or tools have been used to identify adequate risk management measures (RMMs) and operational conditions (OCs):

- Safety data sheets of raw materials

- Identified uses

- ECETOC TRA and/or industry best practices for OCs and RMMs

Typical values for operational conditions and existing risk management measures can be found on the ATIEL Website under: http://www.atiel.org/Reach_GES.htm

| 8 | Generate exposure scenario of mixture or integrate information into chapter 1 – 16 of the SDS |
| --- | --- |

8.1/8.2 	Compile and consolidate risk management measures and operational conditions

In order to generate the exposure scenario for the mixture, the relevant pieces of information from the exposure scenarios of the identified lead substances have been taken and combined to the exposure scenario of the mixture.

It might be that information from exposure scenarios of substances has to be adapted to the specific conditions of use in the mixture. See  Part III, chapter 6.3 of the Practical Guide for details.

The following Risk Management Measures (RMM’S) and Operational Conditions (OC’s) have been adapted from the raw material Safety Data Sheet for ZDDP and integrated into the Safety Data Sheet of the formulated Engine Oil:

Part 6:	Accidental Release Methods: Environmental Precautions

Part 7:	Handling and Storage:  Handling and Storage Temperatures

Part 8:	Exposure Controls / Personal Protection

- Hygiene Measures

- Hand Protection

- Eye Protection

- Environmental Exposure Controls

In the case of the engine oil, the identified use of all substances in the mixture has been adequately covered by the exposure scenarios of the substances received from the suppliers.

At the time, when this example was created, the relevant DNEL’s and PNEC’s of the various raw materials were not available yet.

8.3 Check whether identified risk management measures for lead substances cover other substances in the mixture

Priority substances require most stringent risk management measures. However, if these measures are substance-specific (e.g. a precipitation step), it is possible that they do not control adequately further other substances of the mixture with different physicochemical properties. Therefore it is recommended to apply control steps to ensure that the recommended risk management measures are really adequate for the mixture. These steps are shown in Flow Chart 3 in Part III, chapter 7.3 of the REACH Practical Guide.

The recommended Risk Management Measures and Operational Conditions for the Lead Substance covered recommended RMM’s and OC’s of the other raw materials. There was no need to adjust them

8.4	Decide on scaling guidance and – if yes – develop this guidance

Scaling can broaden the range of applications which are covered by an exposure scenario. Scaling means the use of simple equations in the exposure scenario by which the downstream user can demonstrate that he operates within the conditions of the exposure scenario provided by the registrant. It is described in Part III, chapter 7.7 of the REACH Practical Guide.

We did not provide any scaling guidance because the RMM’s and OC’s we recommend are adequate to ensure safe use in any case for the identified uses.

8.5	Decide in which form the information should be included in the safety data sheet     of the mixture

Downstream users have to include in their own safety data sheets information which they have received from their suppliers (REACH Art. 31.7, see chapter 7.2). They can do this in several ways. They can forward exposure scenarios, they can consolidate information (into an exposure scenario for the mixture) or they can integrate information into chapter 1 – 16  of the safety data sheet (see chapter 7.6 of the REACH Practical Guide).

In the case of the engine oil, inclusion of information has been done in the following way:

- Integration of information into chapter 1 – 16 of the extended safety data sheet;

We decided to integrate the relevant information into chapter 1-16 of the extended safety data sheet because the RMM’s and OC’s describe adequate controls for the safe handling and use of engine oils in general. The provision of a separate exposure scenario was not considered to be beneficial for the customers / users.

8.5	Document the result: exposure scenario and / or extended safety data sheet of the mixture

The safety data sheet for the lead substance is documented in Annex 2.

The safety data sheet for the engine oil is documented in Annex 3.

Annexes:

Annex 1: Lead Substance Identification

Annex 2: Safety Data Sheet for the Lead Substance

Annex 3: Safety Data Sheet for Engine Oil

At the time, when the Safety Data Sheet for the Lead Substance was created, the relevant PNEC was not yet available and could therefore not be used in the generation of the Safety Data Sheet of the Engine Oil (mixture).